IEEE 754
Floating-Point Adder

Verilog RTL Design • Single-Precision (32-bit)

Project overview, architecture, module descriptions,
Vivado simulation workflow, testing, and limitations

Prepared as a concise project report

# 1. Project Overview

The IEEE 754 Floating-Point Adder is a combinational digital circuit designed to add two 32-bit single-precision floating-point numbers. Written in Verilog RTL, it is organized into small modules so the arithmetic stages can be studied and simulated separately.

## Objectives

Accept two 32-bit inputs, a[31:0] and b[31:0].

Split each input into sign, exponent, and fraction fields.

Align exponents before adding or subtracting significands.

Normalize and round the intermediate result, then pack it into a 32-bit output.

Use a testbench and Vivado behavioral simulation to inspect results.

# 2. IEEE 754 Single-Precision Format

A 32-bit number is divided into three fields:

| Field | Width | Purpose |
| --- | --- | --- |
| Sign | 1 bit | 0 = positive; 1 = negative. |
| Exponent | 8 bits | Biased exponent; bias = 127. |
| Fraction | 23 bits | Fraction bits; normalized values have an implicit leading 1. |

For normalized values:

Value = (−1)^sign × 1.fraction × 2^(exponent − 127)

Exponent encodings also represent zero/subnormal values, infinity, and NaN. Full support for these cases is not implemented in the starter RTL.

# 3. Architecture and Processing Flow

The top module connects the stages in this order:

| Stage | Operation |
| --- | --- |
| 01 — Unpack | Extract sign, exponent, and fraction; form significand. |
| 02 — Exponent alignment | Shift the smaller significand to a common exponent. |
| 03 — Add/subtract | Add for equal signs or subtract for different signs. |
| 04 — Normalize | Adjust significand and exponent. |
| 05 — Round | Reduce extra precision to output fraction width. |
| 06 — Pack | Combine sign, exponent, and fraction into 32-bit output. |

Top-level interface: Inputs: a[31:0], b[31:0]  |  Output: result[31:0]

# 4. RTL Module Descriptions

01_fp_unpack.v — Extracts sign, exponent, and fraction. Normalized inputs get the implicit leading 1.

02_exp_align.v — Compares exponents and right-shifts the smaller significand; shifted-out bits should be retained for rounding.

03_significand_add.v — Adds magnitudes for equal signs or subtracts for different signs, then selects a result sign.

04_normalize.v — Adjusts the intermediate significand and exponent after arithmetic.

05_round_mantissa.v — Rounds the significand to the fraction width using extra precision available in the prototype.

06_fp_pack.v — Assembles sign, exponent, and fraction into a 32-bit word.

07_fp_adder_top.v — Connects the arithmetic stages and exposes the input/output ports.

08_tb_fp_adder.v — Testbench that applies sample vectors and displays results for simulation.

# 5. Testing and Simulation

Ordinary positive addition, such as 1.5 + 2.25 = 3.75.

Negative operands and mixed-sign subtraction.

Large exponent differences and significand shifts.

Addition carry-out and normalization after subtraction.

Zero, subnormal, infinity, and NaN cases when the implementation is extended.

The starter testbench displays inputs and outputs; it does not guarantee correctness. Add expected-result comparisons and PASS/FAIL assertions. Compare hexadecimal outputs with a trusted IEEE 754 reference.

# 6. Vivado Workflow

Create a new Vivado project using the RTL project flow.

Add files 01_fp_unpack.v through 07_fp_adder_top.v as Design Sources.

Add 08_tb_fp_adder.v as a Simulation Source.

Set fp_adder_top as the design top module.

Set tb_fp_adder as the simulation top module.

Run Behavioral Simulation and inspect the waveform and console.

If compilation fails, check that required modules are included, module names match their instantiations, and the correct top module is selected.

# 7. Limitations and Future Improvements

Important: this is an educational prototype, not a fully IEEE 754-compliant adder. Special values are simplified, subnormal support is incomplete, and rounding or edge-case handling may be incorrect.

## Suggested improvements

Implement full guard, round, and sticky (GRS) bit handling.

Improve normalization after cancellation in subtraction.

Handle subnormals, signed zero, infinity, NaN, overflow, and underflow explicitly.

Add selectable rounding modes and automated PASS/FAIL regression tests.

Compare results against a trusted IEEE 754 reference and review synthesis results.

# 8. Conclusion

This project demonstrates the main stages of floating-point addition in a modular Verilog design. The ordered RTL files and top-level module provide a useful starting point for learning and simulation. Comprehensive verification and correct handling of IEEE 754 corner cases are necessary before claiming full compliance.